** NOTE TO SPECIFIER **  Golterman & Sabo; acoustical wall panels, ceiling products, sound diffusers and reflectors.

.

This section is based on the products of Golterman & Sabo, which is located at:

3555 Scarlet Oak Boulevard

St. Louis, MO  63122

Tel:  (800) 737-0307

Tel: (636) 225-8800

Fax: (636) 225-2966

Email: inquiry@golterman.com

www.golterman.com

.

Golterman & Sabo makes a wide variety of acoustical control products. In addition to wall and ceiling panels, banners, and suspended "clouds" that are used for sound absorption, they also produce wall and ceiling sound diffusers and sound reflectors.

.

SECTION 09800 - ACOUSTICAL TREATMENT, Copyright 2002, ARCAT, Inc.

- GENERAL

  - SECTION INCLUDES

** NOTE TO SPECIFIER **  Delete items below not required for project.

    - Ceiling baffles.

  - RELATED SECTIONS

** NOTE TO SPECIFIER **  Delete any sections below not relevant to this project; add others as required.

    - Section 05310 - Steel Decking:  Acoustical steel deck.

    - Section 09111 - Non-Loadbearing Metal Framing:  Ceiling suspension systems.

    - Section 09260 - Gypsum Board Assemblies.

    - Section 09511 - Suspended Acoustical Ceilings:  Conventional grid-supported acoustic ceilings.

    - Section 09512 - Adhesive Applied Acoustical Ceilings.

    - Section 09900 - Paints and Coatings.

  - REFERENCES

** NOTE TO SPECIFIER **  Delete references from the list below that are not actually required by the text of the edited section.

    - ASTM C 423 - Standard Test Method for Sound Absorption and Sound Absorption Coefficients by the Reverberation Room Method; 2000.

    - ASTM E 84 - Standard Test Method for Surface Burning Characteristics of Building Materials; 2000a.

  - PERFORMANCE REQUIREMENTS

** NOTE TO SPECIFIER **  Delete the following paragraph if only tackable panels are specified.

    - Acoustical Absorption:  Perform testing in accordance with ASTM C 423, Type A mounting method unless otherwise specified.

    - Flame Spread Rating:  Provide all components with Class A flame spread rating when tested in accordance with ASTM E 84, unless otherwise specified.

  - SUBMITTALS

    - Submit under provisions of Section 01300.

    - Product Data:  Manufacturer's data sheets on each product to be used, including:

      - Preparation instructions and recommendations.

      - Storage and handling requirements and recommendations.

      - Installation methods.

      - Independent testing agency test reports.

** NOTE TO SPECIFIER **  Delete selection samples if colors have already been selected.

    - Selection Samples:  For each product specified, two complete sets of color samples representing manufacturer's full range of available colors and patterns.

    - Verification Samples:  For each product specified, two samples, minimum size 6 inches (150 mm) square, representing actual product, color, and patterns.

SYSTEM DESCRIPTION

Design Requirements: Baffles shall absorb sound via MDF porous core and 20mm Mineral Wool absorber sandwiched within baffle. High frequency absorption shall be determined by the percent open area and mid frequency absorption shall be determined by the thickness of the diaphragmatic surface template. Baffles shall be AAM Boomerang Ref 28257. 38mm (1-1/2”) thick overall. Face Perforations shall run to the edges of the baffles (no solid borders). Baffles shall be faced with Real Wood Veneers LLC , Baltic Birch FC, Tint # 22289. Baffles shall be fastened with 75x75 HSS Frame with intermittent rods fastened to underside of concrete slab

B.	 Performance Requirements

  - Tested by an independent accredited,facility according to ASTM C 423.

Substrate		250 Hz	500 Hz	1000 Hz	2000 Hz	4000 Hz

- Thickness

38 mm		0.61	0.96	0.95	0.72	0.47

3.	Noise Reduction Coefficient (NRC): Tested by independent, accredited facility according to ASTM C 423.

- Noise Reduction Coefficient = 0.80

  - QUALITY ASSURANCE

    - Manufacturer Qualifications:  Minimum 3 years of experience in producing acoustical products of the types specified herein.

    - Installer Qualifications:  Acceptable to the manufacturer of the acoustical products being installed.

** NOTE TO SPECIFIER **  Include a mock-up if the project size and/or quality warrant taking such a precaution.  The following is one example of how a mock-up on a large project might be specified.  When deciding on the extent of the mock-up, consider all the major different types of work on the project.

    - Mock-Up:  Provide a mock-up for evaluation of installed appearance.

      - Install acoustical products in areas designated by Architect.

      - Do not proceed with remaining work until Architect approves workmanship and appearance.

      - Approved mock-up may remain as part of the work.

  - DELIVERY, STORAGE, AND HANDLING

    - Protect acoustical products from moisture during shipment, storage, and handling.

    - Store products in manufacturer's unopened packaging until ready for installation.

      - Store materials flat, in dry, well-ventilated space.

      - Do not stand panels on end.

      - Protect edges from damage.

    - Store and dispose of solvent-based materials, and materials used with solvent-based materials, in accordance with requirements of local authorities having jurisdiction.

  - PROJECT CONDITIONS

    - Do not begin installation of acoustical products until building has been enclosed and environmental conditions approximate those that will prevail when building is occupied.

    - Environmental Requirements:  Do not install panels until wet work, such as concrete and plastering, is complete; the building is enclosed; and the temperature and relative humidity are stabilized at 60 – 80 degrees F (16 – 27 degrees C) and 40% to 50%, respectively. FOLLOW AWI GUIDELINES.

  - EXTRA MATERIALS

** NOTE TO SPECIFIER **  Include this article if the project size and/or quality warrant the additional expense and the Owner has adequate storage space available for this purpose.  The following is one example of how extra materials on a large project might be specified.  Edit the percentage as appropriate for the project, or change the text to require a specific number.

    - See Section 01600 - Product Requirements, for additional provisions.

    - Provide 5 percent, but not less than 1 of each type of acoustical unit actually installed, for Owner's use in maintenance.

- PRODUCTS

  - MANUFACTURERS

  - American Architectural Millwork LLC, 248 James Street, Venice, FL34285.  Tel: 844-307-9571. www.millworkusa.com.

  - Distributed by ___________________________________________________________

** NOTE TO SPECIFIER **  Delete one of the following two paragraphs; coordinate with requirements of Division 1 section on product options and substitutions.

    - Substitutions:  Not permitted.

    - Requests for substitutions will be considered in accordance with provisions of Section 01600.

    - Provide all acoustical products specified herein by a single manufacturer.

  - CEILING PRODUCTS

    - Acoustical Baffles:  Wrapped panels, with peforated MDF skins around an absorber filled core. Real Wood Veneers LLC , Baltic Birch FC, Tint # 22289 seamless veneer bonded to all exposed faces. Faces micro perforated to the edge.

** NOTE TO SPECIFIER **  Delete one of the following two paragraphs.  Thicknesses up to 4 inches are available.

      - Core Thickness:  1-1/2 inch (38 mm).

** NOTE TO SPECIFIER **  Delete one of the following two paragraphs.

      - Type:  AAM Boomerang Baffle. Perforation 1.8, 1.8, 0.5mm Micro

** NOTE TO SPECIFIER **  Delete all but one finish material below.

      - Finish Material:  Real Wood Veneers LLC Baltic Birch FC, Tint #22289.

** NOTE TO SPECIFIER **  Delete two of the following three paragraphs.

        - Edges:  Square and veneered as face.

      - Color:  Tint # 22289.

  - ACCESSORIES

    - Mounting: 75x75 HSS Frame with intermittent rods supplied by installer

- EXECUTION

  - EXAMINATION

    - Do not begin installation until substrates have been properly prepared.

    - If substrate preparation is the responsibility of another installer, notify Architect of unsatisfactory preparation before proceeding.

  - PREPARATION

    - Clean surfaces thoroughly prior to installation.

    - Prepare surfaces using the methods recommended by the manufacturer for achieving the best result for the substrate under the project conditions.

  - INSTALLATION

    - Install acoustical units in accordance with manufacturer's instructions.

  - PROTECTION

    - Protect installed products until completion of project.

    - Touch-up, repair or replace damaged products before Substantial Completion.

END OF SECTION